# Construction Research: Making the Pedal Enclosure

Oct 30, 2026 · Dallas Caley

A bare metal pedal box has no holes for switches, controls or lights. The simplest route is to buy the bare box, send a hole layout to a drilling service, and get it back finished. You can also drill it yourself, and part of that work is already done. Flashover has a drilling guide in pedals/distortion/kicad/FLASHOVER_STATUS.md and a full-size printable template, pedals/distortion/kicad/Flashover_drill_template.pdf, with positions taken from the board design.

Terms in bold are explained in the Glossary at the end.

# Ways to make the holes

| Method | How it works | Best for | Main catch |
| --- | --- | --- | --- |
| By hand | Tape on the template, centre-punch, then drill with a drill press and step bits, and clean up the edges | One or two prototypes | Hard to keep neat rows of holes |
| Drilling service | Upload hole positions and sizes; a CNC machine drills them, with coating and printing as options | Prototypes through small runs | Limits on accuracy, and some services have minimum orders |
| Pre-drilled box | Buy a box with a standard layout | Simple 2–4 knob pedals | No use for Flashover's layout |
| Your own CNC work | Buy or rent time on a small CNC machine | Regular small batches | Machine cost and setup time |
| Bent sheet metal | Holes are laser-cut while the metal is flat, then it's folded into a box | A custom shape without tooling | Needs a new design, and it's a different look and feel |
| Custom die-cast box | A factory makes a mould for your exact shape | High volume | Tooling is expensive, likely thousands of dollars or more (Claude's estimate) |

# Drilling services

- Tayda: this looks like the obvious starting point, since we already use them. Search results show about $4.50 per 125B box with bulk prices down to about $2, extra holes beyond 40 for a small fee, and 4–5 business days. They also show a 3 mm minimum hole and a tolerance of ±0.5 mm on the top and ±1 mm on the sides.

- Other services: Amplify Fun, F5 Metalworks and Obscura MFG are listed for drilling and finishing, and Obscura has a 25-piece minimum.

Tayda's page blocked direct access, so those figures come from search summaries and may be out of date. It also isn't confirmed that Tayda drills the 1590BB, the box Flashover uses.

# Order of work

- Drill the holes and clean up the edges.

- Apply the powder coat or other finish.

- Print the artwork, for example with UV printing.

- Fit the parts.

Because coating narrows the holes, they need to be drilled slightly oversize. Tayda's example is 9.2 mm for a 9 mm hole.

# Where Flashover makes this harder

- It has a lot of holes: there are 47 in total. The top has 3 knobs, 33 ring LEDs, 4 other LEDs, the button and the footswitch, and the walls have 5 jacks. The ring LEDs are the hard part, because small errors show up in a neat circle, and that's difficult by hand.

- The knob holes are a tight fit: the holes are 7.5 mm for a 7 mm collar, leaving only 0.25 mm per side. A ±0.5 mm tolerance across three encoders on one board could stop the panel from fitting, and coating makes it tighter. Those holes probably need to be bigger.

- The jack pockets may be a problem: the 9V and two MIDI jacks need shallow, flat-bottomed pockets, and it isn't known whether these services will cut them. If not, they'd be cut by hand with a drill press and depth stop.

- The walls are slanted: the box walls have draft, so a wall needs a shim to sit level for drilling by hand.

# Recommendation

- First prototype: get the panel holes drilled by a service, but leave the box bare. Mistakes are then cheap, and the fit can be tested before paying for a finish. Any pockets would be cut by hand.

- Before ordering: settle ring LED holes versus light pipes, the footswitch hole and the button hole, all still marked as undecided or unchecked on the template. Then widen the knob holes to suit the service's tolerance.

- For sale: use a service for drilling, finish and printing, and ask early about pockets and the 1590BB. Decide on tooling or bent metal only if sales justify it.

To try it by hand, use a separate practice box. A basic drill press and step bits would probably cost about $100–200 (Claude's estimate).

# Sources

- Tayda custom drill service

- Tayda additional holes

- StompBoxParts enclosure services

- PedalPCB forum: drilling and finishing

# Glossary

125B, 1590BB: standard aluminium pedal box sizes. The 125B is a common small pedal box; the 1590BB is a wider one, 119.5 x 94 mm.

Centre-punch: to tap a small dent into the metal at a hole's centre, so the drill bit doesn't wander.

CNC (computer numerical control): a machine that drills or cuts automatically from a file of positions.

Collar: the threaded neck of a knob's encoder that passes through the panel and takes a nut.

Die-cast: made by forcing molten metal into a reusable steel mould.

Draft: a slight taper on a cast box's walls that lets it come out of the mould. The walls aren't quite square to the panel.

Light pipe: a small clear plastic rod that carries an LED's light up to the panel surface.

Pocket: a shallow, flat-bottomed widening of a hole on the outside of the wall, so a plug can seat fully in its jack.

Powder coat: a dry paint powder that is sprayed on and baked into a hard finish.

Step bit: a cone-shaped drill bit with stepped sizes, which cuts clean holes of several sizes in thin metal.

Tolerance: how far a hole may land from its specified position.

UV printing: printing artwork directly onto the box with ink that is hardened by ultraviolet light.